##### PART 1 – Generic Risk Assessment of Course

| Course No: E64/10 | Brief Desc: Stanstead Abbotts - A414 - Harlow |  | Page 1 of 2 |
| --- | --- | --- | --- |
| Date last revision: 26/8/21 |  | Date Reviewed by Courses Sub-committee: 26/8/10 |  |
| START (GR37611)on B181 west of Stanstead St. Margaret's at west end of concrete field entrance, opposite L.P.16 (near Hillside Lane). Proceed west along B181 to A414 (Amwell roundabout) (M). Bear left down A.414 to turn at Eastwick Lodge roundabout (5.24m) (M), at junction with unclassified road to High Wych. Return along A414 to Amwell roundabout and bear left (M) down A1170 to FINISH (GR37114)within 300 yards at 15th curb joint past manhole cover, just beyond L.P.283. a) Take care on return, keep right and straight on along A414 at slip road to Stanstead Abbotts/Roydon (B181). b) Continue down A1170 after finishing, to turn at small roundabout - NO 'U' TURNS ALLOWED. c) No parking at the start. |  |  |  |

NOTES:

- Size of Warning Signs (where specified) relates to the size of the triangle i.e.

6oomm (overall dimensions of sign 34” x 24”)

750mm (overall dimensions of sign 44” x 33”)

where not specified then a 360mm sign (overall dimensions of sign 24” x 19”), or larger, may be used.

- All Marshals, and officials at start and finish should be supplied with and wear high visibility jackets.

- All warning signs should be placed at least 100 yards before the relevant hazard.

- Marshals are only specified where we consider there is a safety requirement. It is the Organisers responsibility to post marshals in other positions where it may be necessary to direct riders on the correct route.

|  |  | ###### Hazard |  | Measures to Reduce Risk to LOW |
| --- | --- | --- | --- | --- |
|  | ###### Location | ###### Details | Risk H/M/L |  |
| 1 | General Requirements | Time of road usage meets CTT Traffic Standards |  | Traffic Counts to be carried out as and when instructed by District Committee. Rider briefing notice when signing on emphasise at ALL RAB’s riders must - Give way to traffic from their right Give a clear signal as to their intended route |
| 2 | Before START | Competitors riding from Event HQ Competitors warming up | M | Instructions on Start Sheet: - Safe routes to START - Prohibited areas for warming up - Warning re “U” turns |
| 3 | Area of START | Competitors waiting to start | L | Cycle Race Warning Signs Instructions on Start Sheet re: Conduct at START |
| 4 | START | Hatched centre line road markings at start Competitor and/or Holder-up falling | M | Event Sec. to take measures to ensure riders waiting to start keep off road No parking at start |
| 5 | Amwell RAB | Riders turn left, 1st exit onto A414 | M | Cycle Race Warning Signs on following approach roads to RAB: - A10 - B1502 - A1170 southbound |
| 5a | A414 eastbound | Dual carriageway | M | Cycle Race Warning Sign (600mm) to be placed on A414 eastbound immediately after Amwell RAB |
| 6 | Bridge over canal | Poor bridge join in road surface | M | Riders briefing to include warning of dangerous join in the road surface. |
| 7 | Works access Slip road on left | Event route has priority | L |  |
| 8 | Slip road entering from B181on left |  | M | Cycle Race Warning Sign (600mm) on slip to A414 |
| 9 | Road on left to Hunsdon & The Hadhams | Event route has priority | M | Cycle Race Warning Sign on road from the Hadhams |
| 10 | Service road to houses on left | Event route has priority | L |  |
| 11 | Lane on left | Event route has priority | L |  |
| 12 | Lane on left from Eastwick | Event route has priority | L |  |
| 13 | TURN Eastwick Lodge rbt. | Road marked prior to rbt. into 2 lanes, inner - High Wych, outer - Harlow | H | Cycle Race Warning Signs on all approach roads to rbt. Marshal in high vis. jacket Note on Start Sheet/riders briefing (Riders will be instructed to look & signal before entering outside (Harlow) lane prior to turn.) |
| 13a | A414 westbound | Dual carriageway | M | Cycle Race Warning Sign (600mm) to be placed on A414 westbound immediately after Eastwick Lodge RAB |
| 14 | Lane on left | Event route has priority | L |  |
| 15 | Service road on left | Event route has priority | L |  |
| 16 | Lane on left | Event route has priority | L |  |
| 17 | Lane on left | Event route has priority | L |  |
| 18 | Slip road on left to B181, Stanstead |  | L |  |
| 19 | Slip road on left, from lorry park |  | L |  |
| 20 | Amwell rbt. | Riders turn left, 1st exit A1170 | M | Cycle Race Warning Signs on approach roads to rbt. as per item 5 |
| 21 | FINISH | Parking of motor vehicles | M | Cycle Race Warning Signs in both directions on A1170 |
| 22 | After finish | Riders to return to HQ | M | Instructions on start sheet/riders briefing that: Riders must continue along A1170 after finish to turn at small roundabout No ‘U’ turns |