|  | # Risk Assessment |
| --- | --- |
| Course/Road(s) Assessed: 10 miles unclassified (Latton) | Course: U48 |
| ## Date of Assessment/Review: 15/3/2023 | Name of Assessor: Peter Garnett (reviewed by Paul Winchcombe as part of RA reassements) |

Course Details: Starting on unclassified road (Driffield), proceed onto RAB of Spine Road (B4696) and A419/417. Two ‘laps’ of Latton road turning at the end of each leg on RAB at (B4696)/A419 and RAB of A419/Marston Meysey. Upon finishing riders are to proceed to RAB of B4696 and follow Spine Road back to Event HQ

Traffic Flows: Single carriageway road – traffic flows both directions, North West/South East

Course/Event History: Event is jointly organised by Swindon R.C. and Corinium C.C. Course has remained unchanged since creation.

Key Identified Risks

| Distance | ### Location | Identified Significant Risk/Hazards | Level of Risk Low/Med/High | #### Measures to reduce Risk (if applicable) |
| --- | --- | --- | --- | --- |
| 0 miles | Before Start | Riders unaware of ‘local’ hazards. Visibility of riders and officials to other road users. Competitors riding from Event HQ. Rider safety | Low | Instructions on Start Sheet and at event HQ signing on advising riders of safe routes; Observers will be required to wear Hi-Viz jackets, competitors are required to wear fluorescent numbers on their back and left arm. All riders must now have working front and rear lights and wear a recognised hardshell helmet |
| 0 miles | Area of Start | Competitors waiting | Low | Instructions on Start Sheet and at event HQ signing on Riders to be aware of making ‘U’ turns, bunching at start and other road users. Cycle Event warning sign |
| 0 Miles | Start | Unclassified road ; Light traffic from Driffield; no Layby, although field entrance offers off-road parking and safe area for club officials | Low | Start Sheet instruction – wait on grass or in single file. |
| 0.7 | Driffield unclassified road | Countrywide Store and Farm, traffic entering/leaving site. As out of opening hours for store there should be no traffic | Low | Instruction on start sheet at event HQ – be aware of traffic entering/leaving store/farm site. |
| 0.9 | A419/B4696 (Spine Road) junction RaB | Traffic joining course from A419 and from B4696 (Spine Road); Riders take first left | Med | 1 Observer Cycle Event warning signs on RaB, facing each joining road |
| 1.6 | Junction to/from Down Ampney | Traffic joining course from Down Ampney or turning into Down Ampney Road | Med | Cycle Event warning sign |
| 2.3-2.7 | Latton | Traffic calming build-outs | Low | Warning on Start Sheet |
| 3.2 | Marston Meysey RaB | Riders turn at RaB to retrace Traffic joining from the Northbound A419 and Marston Meysey road. Traffic may exit the RaB to join A419 Southbound | Med | 1 Observer Cycle Event Warning sign, facing each joining road |
| 3.9-4.3 | Latton | Traffic calming build-outs | Low | Warning on Start Sheet |
| 5.0 | Junction to/from Cerney Wick (Unclassified lane) | Traffic joining course from Cerney Wick or turning into Cerney Wick Lane. Very light traffic volume | Low | Junction sits opposite to Down Ampney Road, Cycle Event Warning signs, already in place |
| 5.3 | Passing finish on first ‘lap’ | Riders finishing, other road users, spectators in lay-by. Road surface: missing cats eyes – small pot holes | Low | Warning on Start Sheet |
| 5.7 | A419/B4696 (Spine Road) junction RaB | Traffic joining course from A419 and from B4696 (Spine Road); Riders retrace – taking 3rd exit | Med | As per |
| 6.4 | Junction to/from Down Ampney | Traffic joining course from Down Ampney or turning into Down Ampney Road | Med | As per |
| 7.1-7.5 | Latton | Traffic calming build-outs | Low | Warning on Start Sheet |
| 8.0 | Marston Meysey RaB | Riders turn at RaB to retrace Traffic joining from the Northbound A419 and Marston Meysey road. Traffic may exit the RaB to join A419 Southbound | Med | As per |
| 8.6-9.0 | Latton | Traffic calming build-outs | Low | Warning on Start Sheet |
| 9.7 | Junction to/from Cerney Wick (Unclassified lane) | Traffic joining course from Cerney Wick or turning into Cerney Wick Lane. Very light traffic volume | Low | As per |
| 10.0 | Finish at lay-by short of Spine Road junction | Riders finishing, other road users, spectators in lay-by. Road surface: missing cats eyes – small pot holes | Low | Number checker needed with timekeeper |
| 10.1 | Finish area | Other road users Riders performing Road surface: missing cats eyes – small pot holes | Low | Warning on start sheet not to perform ‘U’ turns at the finish and of recommended route back to Event HQ. Also a note to be aware of surroundings and other road users at the finish. |

NOTES: 1. The use of a vehicle for the timekeeper(s) at start and finish is where appropriate and is only identified as an example for the action to be taken 2. The small junctions or entrances to farms/ facilities (garage,.eatery, etc) that are not identified in this risk assessment have been considered, however are not considered significant to pose a risk and therefore have not been noted.

Date of original Assessment:   15 Oct 11

Date of review 5 April 2014