London East District

Cycling Time trials is a Company Limited by Guarantee Registered England No. 4413282

PART 1 – Generic Risk Assessment of Course

| Course No: El8/06b | Brief Desc: Circuit- East Hanningfield – Howe Green - East Hannigfield |  |  | Page 1 of 2 |
| --- | --- | --- | --- | --- |
| Date of last revision: 26/8/10 |  |  | Date Reviewed bv Courses Sub-committee: 26/8/10 |  |
| START (GR765015)At the junction of Back Lane and the Common, East Hanningfield. Proceed to Howe Green where turn left onto Southend Road (old A130) continue south on Southend Road to turn left into Pan Lane. Continue to East Hanningfield and through village, left at mini roundabout(appears straight on) junction with Bicknacre Road to start point (5.75miles). Repeat complete circuit up to 2 more times as required. To finish in Old Church Road at the junction to back Lane immediately prior to Ashley Green Road, East Hanningfield. |  |  |  |  |

- Size of Warning Signs (where specified) relates to the size of the triangle i.e.

  - 600mm(overall dimensions of sign34" x24")

  - 750mm(overall dimensions of sign 44" x33")

  - where not specified then a 360mm sign(overall dimensions of sign 24" x 19"), or larger, may be used.

- All Marshals, and officials at start and finish should be supplied with and wear high visibility jackets.

- All warning signs should be placed at least 100 yards before the relevant hazard. At junctions where traffic joins the course then the sign should be placed immediately before the junction.

- Marshals are only specified where we consider there is a safety requirement. It is the Organisers responsibility to post marshals in other positions where it may be necessary to direct riders on the correct route.

|  | Location | Hazard |  | Measures to Reduce Risk to LOW |
| --- | --- | --- | --- | --- |
|  |  | Details | Risk H/M/L |  |
| 1 | General Requirements | Time of road usage meets CTT Traffic Standards | L | Traffic Counts to be carried out as Traffic Standards and when instructed by District Committee |
| 2 | Before START | Competitors riding from Event HQ Competitors warming up | L |  |
| 3 | Area of START | Competitors waiting to start | L | Riders waiting to start should stand in Back Lane adjacent to the Start and off the main road. Cycle Race Warning Sign on The Common Road prior to Start All officials at Start to wear High Vis jackets |
| 4 | START | Competitor and/or Holder-up falling | L |  |
| 5 | Howe Green- Alexander Mews (residential Close) joins on left | Rider has priority, minor road marked with Give Way markings | L |  |
| 6 | Howe Green – Chalklands (residential Close) joins on left | Rider has priority, minor road marked with Give Way markings | L |  |
| 7 | Howe Green-junction with Southend Road (old A130) | Riders turn left | M | Marshal in High Vis jacket Cycle Race Warning Sign on Southend Road prior to Junction |
| 8 | Old Southend road(far end)joins on left | Rider has priority, minor road marked with Give Way markings | L |  |
| 9 | Old Southend road(near end)joins on left | Rider has priority, minor road marked with Give Way markings | L |  |
| 10 | Pan Lane | Rider turn left | L |  |
| 11 | East Hanningfield - Back Lane joins on left | Rider has priority, minor road marked with Give Way markings | L |  |
| 12 | East Hanningfield - Back Lane joins on left (for second time) | Rider has priority, minor road marked with Give Way markings | L |  |
| 13 | FINISH, if Finish go to 21 | Officials on side of road |  | All officials to wear High Vis. Jackets |
| 14 | East Hanningfield – Ashley Green (residential Close) joins on left | Rider has priority, minor road marked with Give Way markings | L |  |
| 15 | East Hanningfield - Car Park to Windmill Tavern on left |  | L |  |
| 16 | East Hanningfield – Traffic Calming |  | M | Cycle Race warning sign prior to width restriction facing on-coming traffic |
| 17 | East Hanningfield – Road to Village Hall/School joins on left | Rider has priority, minor road marked with Give Way markings | L |  |
| 18 | East Hanningfield – Mini RAB |  | L | Start sheet/riders briefing sheet to instruct riders that they must give way to any traffic turning right from Bicknacre Road. |
| 19 | East Hanningfield – Abbeyfields (residential Close) joins on left | Rider has priority, minor road marked with Give Way markings | L |  |
| 20 | East Hanningfield – Back Lane joins on left | Rider has priority, minor road marked with Give Way markings | L |  |
| 21 | If more than one circuit repeat items 5 to 19 |  |  |  |
| 22 | After Finish | Riders returning to HQ | L | Start sheet/riders briefing sheet to instruct riders to take care when riding back to HQ, that they may be passed by other riders still completing the event. |