##### PART 1 – Generic Risk Assessment of Course

| Course No: E16/10 | Brief Desc: Stapleford Abbotts - B175 - Marden Ash |  |  | Page 1 of 2 |
| --- | --- | --- | --- | --- |
| Date of last revision: 26/8/21 |  |  | Date Reviewed by Courses Sub-committee: 26/8/10 |  |
| START (GR 499958) at Stapleford Abbotts on the B175, at the junction with Hook Lane, at the northern end of the bus lay-by (at third kerb joint from broken paint line which indicates kerb line across Hook Lane. Proceed north along B175 and A113 to Marden Ash roundabout (5.48m), where turn WITH CARE (M & C). Retrace along A113 and B175 to FINISH (503973) at drain grid past entrance to telephone exchange, which is 45 yards past L.P.1 at centre of bus lay-by immediately after Passingford Bridge. |  |  |  |  |

- NOTES:	Size of Warning Signs (where specified) relates to the size of the triangle i.e.

600mm (overall dimensions of sign 34” x 24”)

750mm (overall dimensions of sign 44” x 33”)

where not specified then a 360mm sign (overall dimensions of sign 24” x 19”), or larger, may be used.

All Marshals, and officials at start and finish should be supplied with and wear high visibility jackets.

All warning signs should be placed at least 100 yards before the relevant hazard.

Marshals are only specified where we consider there is a safety requirement. It is the Organisers responsibility to post marshals in other positions where it may be necessary to direct riders on the correct route.

|  |  | ###### Hazard |  | Measures to Reduce Risk to LOW |
| --- | --- | --- | --- | --- |
|  | ###### Location | ###### Details | Risk H/M/L |  |
| 1 | General Requirements | Time of road usage meets CTT Traffic Standards |  | Traffic Counts to be carried out as and when instructed by District Committee. Rider briefing notice when signing on emphasise at ALL RAB’s riders must - Give way to traffic from their right - Give a clear signal as to their intended route |
| 2 | Before START | Competitors riding from Event HQ Competitors warming up | L | Instructions on Start Sheet: - Safe routes to START - Prohibited areas for warming up - Warning re “U” turns |
| 3 | Area of START | Competitors waiting to start | L | Cycle Race Warning Signs Instructions on Start Sheet re: Conduct at START |
| 4 | START | Competitor and/or Holder-up falling | L |  |
| 5 | Hook Lane jct. | Event route has priority with “Give Way” road markings | L |  |
| 6 | (B175) | Bollard in centre of road, kerb recessed to accommodate traffic | L |  |
| 7 | Passingford Bridge rbt. | Riders cross rbt. 2nd exit A113 (N) | M | Cycle Race Warning Sign on A113 from Abridge |
| 8 | Road on left, from Epping | Event route has priority with “Give Way” road markings | L |  |
| 9 | Road on left, from Stapleford Tawney | Event route has priority with “Give Way” road markings | L |  |
| 10 | Road on left, (Berwick Lane) from Toot Hill | Event route has priority with “Give Way” road markings | L |  |
| 11 | Lay-by on left | Large lay-by with ent/exit | L | . |
| 12 | Road on left, from Stanstead Rivers | Event route has priority with “Give Way” road markings | L |  |
| 13 | Road on left, (Church Road) from Stanstead Rivers | Event route has priority with “Give Way” road markings | L |  |
| 14 | TURN Marden Ash rbt. | Riders circle rbt. 3rd exit to rejoin A113 (S) | M | Direction Sign on approach to rbt Cycle Race Warning Sign on A128 and A113 approaches to rbt. Marshal in high vis. jacket |
| 15 | Lane on left, from Littlebury Hall | Event route has priority with “Give Way” road markings | L |  |
| 16 | Little End, Piggotts factory on left | Ent/exit | L |  |
| 17 | Littlebury, (Strethall Rd) | Ent/exit | L |  |
| 18 | Lane from ex ‘White Bear’ PH | Event route has priority with “Give Way” road markings | L |  |
| 19 | Woodman PH | Event route has priority with “Give Way” road markings | L |  |
| 20 | Road on left, (Shonks Mill Road) | Event route has priority with “Give Way” road markings | L |  |
| 21 | Suttons health farm | residential with ent/exit Event route has priority with “Give Way” road markings | L |  |
| 22 | Passingford Bridge rbt. | Riders cross rbt. 1st exit B175 | M | Cycle Race Warning Sign on A113 from Abridge |
| 23 | FINISH | Parking of motor vehicles | L | Timekeepers only parking on grass verge |
| 24 | After finish | Riders return to HQ | L |  |