##### PART 1 – Generic Risk Assessment of Course

| Course No: E22/10b | Brief Desc: Peldon - Abberton - Peldon |  |  | Page 1 of 1 |
| --- | --- | --- | --- | --- |
| Date of last revision: 2/2/05 |  |  | Date Reviewed by Courses Sub-committee: 16/06/03 |  |
| START (GR 001179) on Peldon road, southwest of Abberton, at paint mark 580 yards southwest of sign 'Peldon'. Proceed southwest to Peldon village, where turn left into St Ives Road, then left at triangle (CARE) into Mersea Road. Turn left at Rose PH to Abberton, where turn sharp left at Lion PH and immediately left again into Peldon Road. Continue through start (5.2m) and repeat circuit to FINISH (GR 004183) at 'Peldon' sign on Peldon road. |  |  |  |  |

NOTES:	All Marshals should be supplied with high visibility marshalling jackets.

All warning signs should be placed at least 100 yards before the relevant hazard.

Marshals are only specified where we consider there is a safety requirement. It is the Organisers responsibility to post marshals in other positions where it may be necessary to direct riders on the correct route.

|  |  | ###### Hazard |  | Measures to Reduce Risk to LOW |
| --- | --- | --- | --- | --- |
|  | ###### Location | ###### Details | Risk H/M/L |  |
| 1 | General Requirements | Time of road usage meets CTT Traffic Standards |  | Traffic Counts to be carried out as and when instructed by District Committee. |
| 2 | Before START | Competitors riding from Event HQ Competitors warming up | L | Instructions on Start Sheet: - Safe routes to START - Prohibited areas for warming up - Warning re “U” turns |
| 3 | Area of START | Competitors waiting to start Horse riders in area of Start | M | Cycle Race Warning Signs Note of caution in riders briefing Instructions on Start Sheet re: Conduct at START |
| 4 | START | Competitor and/or Holder-up falling | L |  |
| 5 | Peldon, left turn, (St Ives Road) | Riders turn left | L |  |
| 6 | Peldon, left turn, (Mersea Road) | Riders turn left & must give way to traffic from right | M | Cycle Race Warning Sign Marshal in high vis jacket |
| 7 | Peldon Rose PH. Left turn to Abberton | Riders turn left & must give way to traffic from right | M | Cycle Race Warning Sign Marshal in high vis jacket |
| 8 | Abberton, left turn | Riders turn sharp left at Lion PH. | M | Cycle Race Warning Sign Marshal in high vis jacket |
| 9 | Abberton, left turn to Peldon | Riders turn left at Fox PH. Riders must give way to traffic from right | M | Cycle Race Warning Sign Marshal in high vis jacket |
| 10 | FINISH |  | L | Cycle Race Warning Signs |