##### PART 1 – Generic Risk Assessment of Course

| Course No: E7/25c | Brief Desc: Tendring - - Thorpe Green |  |  | Page 1 of 2 |
| --- | --- | --- | --- | --- |
| Date of last revision: 17/3/19 |  |  | Date Reviewed by Courses Sub-committee: |  |
| START (TM143244) at Red paint spot on kerb, adjacent to lowered kerb/field entrance, opposite footpath sign at Tendring Village end of School Road. Proceed through Tendring Village on Thorpe Road and turn left into Swan Road (1.45m). Continue through Beaumont (Swan Road becomes Chapel Road) and turn left onto B1414 (2.85m). Continue on B1414 and turn left into Wix Road (3.20m). Continue on Wix Road (Wix Road becomes Clacton Road) through Stones Green to turn left into Colchester Road (4.6m). Turn Right to remain on Colchester Road (4.75m) and continue (Colchester Road/Tendring Road/Stones Green Road) to Tendring Heath (6.65m). Turn sharp left into Heath Road and continue through Tendring Green (Heath Road becomes School Road) to the start (8.40m). Repeat loop 2 further times to FINISH (TM145246) at 30mph repeater signs, after Tendring School, adjacent lowered curb/field entrance. (25.00m) |  |  |  |  |

- NOTES:	Size of Warning Signs (where specified) relates to the size of the triangle i.e.

600mm (overall dimensions of sign 34” x 24”)

750mm (overall dimensions of sign 44” x 33”)

where not specified then a 360mm sign (overall dimensions of sign 24” x 19”), or larger, may be used.

All Marshals, and officials at start and finish should be supplied with and wear high visibility jackets.

All warning signs should be placed at least 100 yards before the relevant hazard.

Marshals are only specified where we consider there is a safety requirement. It is the Organisers responsibility to post marshals in other positions where it may be necessary to direct riders on the correct route.

|  |  | ###### Hazard |  | Measures to Reduce Risk to LOW |
| --- | --- | --- | --- | --- |
|  | ###### Location | ###### Details | Risk H/M/L |  |
| 1 | General Requirements | Time of road usage meets CTT Traffic Standards |  | Traffic Counts to be carried out as and when instructed by District Committee. Rider briefing notice when signing on emphasise at ALL RAB’s riders must - Give way to traffic from their right - Give a clear signal as to their intended route |
| 2 | Before START | Competitors riding from Event HQ Competitors warming up | L | Instructions on Start Sheet: - Safe routes to START - Prohibited areas for warming up - Warning re “U” turns |
| 3 | Area of START | Competitors waiting to start | L | Cycle Race Warning Signs Instructions on Start Sheet re: Conduct at START |
| 4 | START | Competitor and/or Holder-up falling | L |  |
| 5 | Tendring Village | Houses and Church | L |  |
| 6 | Crow Lane on Right | Course has right of way | L |  |
| 7 | Left turn into Swan Road | Course leaves main road | L |  |
| 8 | Left turn onto B1414 | Course joins main road | M | Cycle race warning sign on B1414 to right of junction |
| 9 | Left turn into Wix Road | Course leaves main road | L |  |
| 10 | Wix Road on right | Course has right of way | L |  |
| 11 | Stonehall Lane on left | Course has right of way | L |  |
| 12 | Heath Road (lane) on left | Course has right of way | L |  |
| 13 | T junction with Heath Road | Sharp left turn onto major road | M | Cycle race warning sign on B1035 to right of junction |
| 14 | Repeat item 5 to 13 twice more |  |  |  |
| 16 |  |  |  |  |
| 19 | FINISH | Parking of motor vehicles | L | Cycle Race Warning Signs before and after finish. Timekeepers only parking |
| 20 | After Finish | Riders return to HQ | L | Directions at sign on sheet |