##### PART 1 – Generic Risk Assessment of Course

| Course No: E2/07 | Brief Desc: Newton – Whittlesford – Little Shelford |  |  | Page 1 of |
| --- | --- | --- | --- | --- |
| Date of last revision: 26/8/22 |  |  | Date Reviewed by Courses Sub-committee: 26/8/22 |  |
| START in Newton village, just prior to the entrance of Kidmans Close on Whittlesford Rd proceed eastwards and turn left at the T-junction onto Shelford Rd (1.9mi), continue for 1.5mi then take the left fork onto High St into Little Shelford (3.9mi), then turn left onto Newton Rd at the far end of the the village (4.2mi). Turn left at the T-junction onto the B1368 (5.0mi) then continue for 0.8mi to the finish at the side of Newton Green opposite the bus stop approx 50yds short of Whittlesford Rd (5.8mi). |  |  |  |  |

- NOTES:	Size of Warning Signs (where specified) relates to the size of the triangle i.e.

600mm (overall dimensions of sign 34” x 24”)

750mm (overall dimensions of sign 44” x 33”)

where not specified then a 360mm sign (overall dimensions of sign 24” x 19”), or larger, may be used.

All Marshals, and officials at start and finish should be supplied with and wear high visibility jackets.

All warning signs should be placed a minimum of 50 yards before the relevant hazard.

Marshals are only specified where we consider there is a safety requirement. It is the Organisers responsibility to post marshals in other positions where it may be necessary to direct riders on the correct route.

|  |  | ###### Hazard |  | Measures to Reduce Risk to LOW |
| --- | --- | --- | --- | --- |
|  | ###### Location | ###### Details | Risk H/M/L |  |
| 1. | General Requirements | Time of road usage meets CTT Traffic Standards |  | Traffic Counts will be taken as and when directed by District Committee |
| 2. | Before START | Competitors riding from Event HQ Competitors warming up | L | Instructions on Start/Briefing Sheet: - Safe routes to START - Prohibited areas for warming up - Warning re “U” turns |
| 3. | Area of START | Competitors waiting to start | M | Cycle Race Warning Sign to be placed on brow of railway bridge prior to Newton Rd junction. Instructions on Start/Briefing Sheet re. conduct at START |
| 4 | START | Competitor and/or Holder-up falling | L |  |
| 5 | Kidmans Close | Parked cars on left-hand side of road adjacent to houses. | L | Cycle Race Warning Sign in Whittlesford Rd |
| 6 | Middlemore Rd enters on left | Rider has priority, Give Way road markings | L |  |
| 7 | Whittlesford, left turn at small Village green into Shelford Rd | Rider does not have priority, traffic from right, Give Way line | M | Cycle Race Warning Sign in Shelford Rd 150 yds before junction |
| 8 | Left fork off Shelford Rd into High St. | Rider has right of way | L |  |
| 9 | Left turn into Newton Rd | Rider has right of way | L |  |
| 10 | Left turn into Cambridge Rd | Rider does not have priority, traffic from right, Give Way line | M | Cycle Race Warning Sign to be placed on brow of railway bridge prior to Newton Rd junction. |
| 11 | FINISH | Officials on side of road | L | All officials to wear high vis jackets |
| 12 | After Finishing | Riders turning in road to return to Start/Finish area. | M | Start/Briefing Sheet to instruct riders that they must stop and exercise extreme care before turning. |