|  | # Risk Assessment Type B (Club) Course |
| --- | --- |
| Course Assessed: - E33/10 | Course local to Cambridge Cycling Club |
| ## Updated: - 27 December 2023 | Name of Assessor/Reviewer: Ken Miller/Chris Dyason |

Course Description: START (GR: - TL5395660167) on A1303, 34yds east of the Bell Road corner lamp post. Proceed eastwards to Stetchworth roundabout (RAB) and TURN to re-trace outward route (avoiding A14 slip) to FINISH (GR: - TL5388660175) opposite Bell Road

Notes - Size of Warning Signs (where specified) relates to the size of the triangle i.e. where not specified then a 360mm sign (overall dimensions of sign 24” x 19” SMALL), or larger, may be used. 600mm (overall dimensions of sign 34” x 24” MEDIUM) 750mm (overall dimensions of sign 44” x 33” LARGE) rectangular kickstand sign to be used at the START - All Marshals, and officials at start and finish should be supplied with and wear high visibility jackets. All warning signs should be placed at least 100 yards before the relevant hazard. Marshals are specified where we consider there is a safety requirement and where necessary to indicate the route. It is the organiser’s responsibility to post additional marshals where thought desirable. - The small junctions or entrances to farms/facilities (garage, commercial premises, eatery, etc) that are not identified in this risk assessment have been considered and are considered not to pose a significant risk.

Key Identified Risks

|  | ### Location | Identified Significant Risk/Hazards | Risk L/M/H | #### Measures to reduce Risk (if applicable) |
| --- | --- | --- | --- | --- |
| 1 | General Requirements | Time of road usage meets CTT Traffic Standards |  | Traffic Counts to be carried out as and when instructed by District Committee. |
| 2 | Evening Events – possible low light conditions | Vehicle drivers not observing competitors | M | Competitors encouraged to use a working rear facing light. |
| 3 | Area of START | Competitors warming up Competitors waiting to start | M | Kickstand Cycle Event Warning Sign on A1303 west of Bell Road. LARGE Cycle Event Warning Sign on A1303 east of finish. Cycle Event Warning Sign in Bell Road north of junction with A1303 Instructions on Start Sheet: - Conduct at START - Prohibited areas for warming up - Warning re “U” turns |
| 4 | START | Competitor and/or Pusher falling | L |  |
| 5 | Wilbraham Road | Minor road on right | L |  |
| 6 | Bottisham High Street | Minor road on left | L |  |
| 7 | X roads to Swaffham Bulbeck and Dullingham | Minor cross-roads | L |  |
| 8 | TURN at Stetchworth RAB | Competitors negotiating RAB | M | LARGE Cycle Event Warning Signs prior to RAB on roads not used by event route. LARGE Cycle Event Warning Sign on event route after RAB. Competitor briefing notice when signing on: - - Give way to traffic from right - Signal clearly to other road users - Marshal wearing hi-vis jacket to be stationed on RAB in view of A1304 traffic approaching from Six Mile Bottom. |
| 9 | X roads to Dullingham and Swaffham Bulbeck | Minor cross-roads | L |  |
| 10 | Slip road to A14 | Exit slip road on left | L |  |
| 11 | Bottisham High Street | Minor road on right | L |  |
| 12 | Wilbraham Road | Minor road on left | L |  |
| 13 | FINISH. | Parking of motor vehicles. | M | No parking to be allowed at finish |
| 14 | After finish | Competitors returning to congregation area | M | Instructions in Start Sheet and competitors briefing that there is to be NO STOPPING at finish. |

Signs

Start/Finish – 2 x Kickstand

Bell Lane – 1 x Small

Stetchworth RAB – 4 x Medium

Marshals

Stetchworth RAB x 1

Updated 27 December 2023 verifying the course does not go through a 20 MPH speed limit zone.